CERTIFICATE OF ANALYSIS

| Material/Product Name | Picoxystrobin 6.8 + Tricyclazole 20.33 % SC |  |  |
| --- | --- | --- | --- |
| Batch Number | SAGPT25002 | Quantity | 1317 L |
| Mfg. Date | MAR-2025 | Expiry Date | MAR-2027 |

| S. No. | Test Parameter | Specification | Result |
| --- | --- | --- | --- |
| 1 | Appearance | Material shall be in the form of free-flowing suspension. There may be some clear liquid on the top and/or sediment in the bottom. | Complies |
| 2 | Picoxystrobin Tech. content (% by mass) | 6.80 % (+10 %, -5%) | 6.94 |
| 3 | Tricyclazole Tech. content (% by mass) | 20.33% ( +5%) | 20.56 |
| 4 | pH value (1% aq. Sol.) | 6 – 8 | 6.78 |
| 5 | Sieve Through 45 micron Min | 98.0% | 99.12 |
| 6 | Pourability (Residue) Max | 5.0 % | 4.55 |
| 7 | Susceptibility as Picoxystrobin (% by mass) | 90.0% | 97.82 |
| 8 | Susceptibility as Tricyclazole (% by mass) | 90.0% | 94.52 |
| 9 | Persistent Foam Max in 1 minute | 60 ml | 15 |
| 10 | Spontaneity as Picoxystrobin (% by mass) | 90.0% | 98.58 |
| 11 | Spontaneity as Tricyclazole (% by mass) | 90.0% | 96.58 |
| 12 | Viscosity @ 27°C Spindle 63, 60RPM (CPS) | 250 TO 450 | 642 cps |
| 13 | Density@ 27°C | 1.10 to 1.15 | 1.11 |
| Remarks: The material complies with the above specification |  |  |  |